CPE301 – SPRING 2019

# Design Assignment 2B

Student Name: Benjamin Johnson

Student #: 5003284489

Student Email: johnsb18@unlv.nevada.edu

Primary Github address: https://github.com/johnsb18/ClassRepository

Directory: ClassRepository/DesignAssignments/DA2B

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

; GPIO IRQ_ASM.asm

;

; Created: 3/9/2019 4:58:56 PM

; Author : Benjamin Johnson

; Student #: 5003284489

; Email: johnsb18@unlv.nevada.edu

.org 0			; location for reset

jmp MAIN

.org 0x02		; location for external interrupt INT0

jmp EX0_ISR

MAIN:

ldi R20, HIGH(RAMEND)	; initialize stack

out SPH, R20

ldi R20, LOW(RAMEND)

out SPL, R20

ldi R20, 2		; make INT0 falling edge triggered

sts EICRA, R20

sbi DDRB, 2		; set PORTB.2 as output

sbi PORTB, 2	; set LED off

sbi PORTD, 2	; turn on pull-up

cbi DDRC, 2		; set PORTC.2 as input

sbi PORTC, 2	; enable pull up

ldi R20, 1<<INT0	; enable INT0

out EIMSK, R20

sei				; enable interrupts

WHILE:

jmp	WHILE		; wait for interrupt

EX0_ISR:

in	R21, PORTB	; R21 = PORTB

ldi R22, (1<<2)	; load mask for toggling

eor R21, R22	; toggle LED

out	PORTB, R21	; PORTB = R21

; Delay 20 000 000 cycles

; 1s 250ms at 16 MHz

ldi  r18, 102

ldi  r19, 118

ldi  r20, 194

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

reti			; return from interrupt

--------------------------------------------------------------------------

/*

* GPIO IRQ_C.c

*

* Created: 3/9/2019 1:50:27 PM

* Author : Benjamin Johnson

* Student #: 5003284489

* Email: johnsb18@unlv.nevada.edu

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<2);		// set PORTB.2 for output (LED)

PORTB |= (1<<2);	// set LED low

DDRC &= (0<<2);		// connect PORTC.2 to switch as input

PORTC |= (1<<2);	// enable pull-up

EIMSK = (1<<INT0);	// enable external interrupt 0

EICRA = (1<<ISC01);	// make INT0 falling edge triggered

sei ();				// enable interrupts

while (1) {}		// wait for interrupt

}

ISR (INT0_vect)	{		// ISR for external interrupt INT0

PORTB ^= (1<<2);	// toggle PORTB.2 (LED)

_delay_ms(1250);	// delay for 1.25 sec

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Insert only the modified sections here

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/J47CX-IcE50

- GITHUB LINK OF THIS DA

https://github.com/johnsb18/ClassRepository/tree/master/DesignAssignments/DA2B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Benjamin Johnson