CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Benjamin Johnson

Student #: 5003284489

Student Email:  johnsb18@unlv.nevada.edu

Primary Github address: https://github.com/johnsb18/ClassRepository

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmega328P

- FTDI Chip

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* UART.cpp

*

* Created: 3/27/2019 3:31:10 PM

* Author: Benjamin Johnson

* Student #: 5003284489

* Email: johnsb18@unlv.nevada.edu

*/

#define BAUD 9600

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <util/setbaud.h>

void USART_init(void);					// initializes analog to digital

void USART_tx_string(char*data);		// prints a string

void USART_tx_char (char integer);		// prints a character

char nextLine[] = "\n";					// declare next line string

volatile float usart_temp = 3.14;		// declare float

char outs[20];							// allocate memory space for float

int main(void)

{

USART_init();						// initialize analog to digital

TCCR1B |= (1<<CS12) | (1<<CS10);	// prescaler = 1024

TIMSK1 = (1<<TOIE1);				// enable overflow flag

TCNT1 = 49911;						// reset timer (65535 - 15624)

sei();								// enable interrupts

while (1) {}						// wait for timer interrupt

}

void USART_init (void)

{

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); // 8-bit data

UCSR0B = _BV(RXEN0) | _BV(TXEN0);	// enable RX and TX

}

ISR (TIMER1_OVF_vect)

{

USART_tx_string(nextLine);				// next line

USART_tx_string("This is working!");	// print string

USART_tx_string(nextLine);				// next line

USART_tx_char('3');						// print int

USART_tx_string(nextLine);				// next line

snprintf(outs, sizeof(outs), "%f\r\n", usart_temp);	// store float in outs

USART_tx_string(outs);					// print float

USART_tx_string(nextLine);				// next line

TCNT1 = 49911;							// reset timer

}

void USART_tx_string (char*data)

{

while ((*data != '\0')){			// iterate through entire string

while (!(UCSR0A & (1<<UDRE0)));	// wait for UDRE0 flag to go high

UDR0 = *data;					// set USART I/O data register to string

data++;							// iterate through string

}

}

void USART_tx_char (char integer)

{

while (!(UCSR0A & (1<<UDRE0)));		// wait for UDRE0 flag to go high

UDR0 = integer;					// set USART I/O data register to int as char

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

I was unable to get my terminal to render output.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/johnsb18/ClassRepository/tree/master/DesignAssignments/DA3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Benjamin Johnson