CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Benjamin Johnson

Student #: 5003284489

Student Email:  johnsb18@unlv.nevada.edu

Primary Github address: https://github.com/johnsb18/ClassRepository

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

*I was unable to make a video for my project because for some reason my set up was not working. I was able to confirm my code works by having a friend test it on their machine, and I know my components work because a friend borrowed them. But when I tried to run my code on my components, it did not work. Thus I don’t have a video for this assignment.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328P

TB6612FNG Dual Motor Driver Carrier

Lab board (for VCC input)

DC motor

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* PWM DC Motor.c

*

* Created: 4/10/2019 5:30:46 PM

* Author : Benjamin Johnson

* Student #: 5003284489

* Email: johnsb18@unlv.nevada.edu

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

int togMotor = 0;				// motor on/off toggle

int main(void)

{

DDRD = 0x40;					// PD6 is output

DDRC = 0x02;					// set PC0 as input, PC1 as output for interrupts, INT1 for interrupt

PORTC |= (1<<1);				// enable pull-up

TCCR0A |= (1<<COM0A1)|(1<<WGM01)|(1<<WGM00);	// clear OC0A on compare match, fast PWM

TCCR0B |= (1<<CS02)|(1<<CS00);					// prescaler = 1024

PCICR |= (1<<PCIE1);			// enable pin change interrupt 1

PCMSK1 |= (1<<PCINT9);			// enable pin change on PC1

ADMUX |= (1<<REFS0);			// use ref voltage at Aref

ADCSRA |= (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(ADPS2)|(1<<ADPS1)|(1<<ADPS0);

sei();

while (1) {	}		// run while waiting for interrupt

}

ISR (PCINT1_vect) {		// toggle interrupt

if (!(PINC & (1<<PINC1))) {

if (togMotor == 0) {	// turn motor off

OCR0A = 0;

PORTB &= ~(1<<PORTB2);

}

if (togMotor == 1) {	// turn motor on

while ((ADCSRA & (1<<ADIF)) == 0);	// process ADC

OCR0A = ADC;

PORTB |= (1<<PORTB2);

}

_delay_ms(500);

togMotor ^= 1;		// toggle motor control on/off

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

*Refer to note at the top of the second page.

- GITHUB LINK OF THIS DA

https://github.com/johnsb18/ClassRepository/tree/master/DesignAssignments/DA4A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Benjamin Johnson