CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Benjamin Johnson

Student #: 5003284489

Student Email:  johnsb18@unlv.nevada.edu

Primary Github address: https://github.com/johnsb18/ClassRepository

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328P

Unipolar stepper motor

ULN2003

Servo motor

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Stepper Motor.c

*

* Created: 4/22/2019 4:32:32 PM

* Author : Benjamin Johnson

* Student #: 5003284489

* Email: johnsb18@unlv.nevada.edu

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

int main(void)

{

DDRD = 0x0F;								// PORTD lower pins are output

TCCR1A |= (1<<WGM01);						// CTC mode

TCCR1B |= (1<<WGM12)|(1<< CS12)|(1<<CS10);	// prescaler = 1024

ADMUX |= (1<<REFS0);				// use ref voltage at Aref

ADCSRA |= (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(ADPS2)|(1<<ADPS1)|(1<<ADPS0);

while (1)		// timer set for .1 second (65536 - 1562)

{

while ((ADCSRA & (1<<ADIF)) == 0);	// process ADC

OCR1A = ADC;				// set new limit

TCNT1 = 0;					// reset timer

while (TCNT0 != OCR1A) {}	// delay

PORTD = 0x09;				// rotate motor

TCNT1 = 0;					// reset timer

while (TCNT0 != OCR1A) {}	// delay

PORTD = 0x03;				// rotate motor

TCNT1 = 0;					// reset timer

while (TCNT0 != OCR1A) {}	// delay

PORTD = 0x06;				// rotate motor

TCNT1 = 0;					// reset timer

while (TCNT0 != OCR1A) {}	// delay

PORTD = 0x0C;				// rotate motor

}

}

/*

* Servo Motor.c

*

* Created: 4/22/2019 6:20:52 PM

* Author : Benjamin Johnson

* Student #: 5003284489

* Email: johnsb18@unlv.nevada.edu

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

TCCR1A |= (1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);	// non-inverted PWM

TCCR1B |= (1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10);	// prescaler 64, mode 14 fast PWM

DDRB = 0x02;								// set PB1 to output

DDRD = 0x40;						// PD6 as output

ADMUX |= (1<<REFS0);				// use ref voltage at Aref

ADCSRA |= (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(ADPS2)|(1<<ADPS1)|(1<<ADPS0);

while (1)

{

while ((ADCSRA & (1<<ADIF)) == 0);	// process ADC

OCR1A = ADC;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/johnsb18/ClassRepository/tree/master/DesignAssignments/DA4B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Benjamin Johnson